# Project Black Pearl - Emergency No-Start Fuel Starvation Recovery Guide

# 🚘 Vehicle Info

2005 Subaru WRX EJ205 (non-AVCS) – Project Black Pearl

Symptoms: Cranks, no start, no fuel pump prime, no ignition. CEL present.

# 🧰 Field Diagnostic Checklist

## 1. Bypass Clutch Switch

- Locate the clutch safety switch plug behind the clutch pedal.
- Unplug it.
- Use a jumper wire or paperclip to bridge the two terminals in the harness.
- Attempt to crank.
Result: 
✅ Cranks → Replace or permanently bypass clutch switch.
❌ No crank → Continue to Step 2.

## 2. Direct Power to Fuel Pump

- Remove rear seat bottom.
- Unscrew fuel pump cover (driver side).
- Expose the fuel pump harness.
- Use fused jumper wire: Battery 12V → Red or White/Black wire, Ground → Black wire.
Result:
✅ Pump hums → Pump functional. Move to Step 3.
❌ Silent → Replace pump or inspect wiring.

## 3. Fuel Pump Relay & Fuse Test

- Under-hood fuse box: Pull Fuel Pump Relay.
- Swap with another relay (AC, horn).
- Key ON, listen for 2–3 second hum.
- If no hum, jumper pin 30 → 87 (manual pump test).
Result:
✅ Pump primes on jumper → ECU not commanding pump (see Step 4).
❌ No pump → Ground/fuse/pump issue.

## 4. Crankshaft Position Sensor (CKP) Test

- Located behind crank pulley, lower front of engine.
- Verify plug is fully seated.
- Optional: Test resistance (500–700 ohms typical).
- Use Tactrix or OBD2 reader: crank engine and monitor RPM.
Result:
✅ RPM rises during crank → CKP good.
❌ RPM = 0 → Replace CKP sensor.

## 5. Starter Solenoid Trigger

- Locate solenoid signal wire on starter motor.
- Key ON, bridge battery post to solenoid tab with screwdriver.
Result:
✅ Cranks → Starter good. Issue in ignition/starter relay path.
❌ No crank → Replace starter.

# 🚨 Recovery Sequence

1. Ensure oil level is at full.
2. Jumper clutch switch.
3. Power pump directly to verify function.
4. Log RPM on crank (check crank sensor).
5. Reconnect everything, key ON, and crank.
6. Monitor oil pressure light, idle sound, CEL.